SECTION 062020

EXTERIOR PVC RAILINGS

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and other Contract Documents, listed in the agreement between the Owner and Contractor, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Exterior PVC railings.

      - ACTION SUBMITTALS

        - Product Data: For each type of process and factory-fabricated product. Indicate component materials, dimensions, profiles, and colors and include construction and application details.

        - Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

        - Samples for Initial Selection: For each type of product involving selection of colors, profiles, or textures.

        - Samples for Verification:

          - For cellular PVC railing components, with half of exposed surface finished; 50 sq. in.

        - Delegated-Design Submittal: For railing systems, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

      - INFORMATIONAL SUBMITTALS

        - Evaluation Reports: For the following, from ICC-ES:

          - Cellular PVC.

        - Warranties: Provide sample warranties.

      - QUALITY ASSURANCE

        - Engineer PVC railing system to withstand design loads indicated on Drawings.

      - DELIVERY, STORAGE, AND HANDLING

        - Store materials flat with spacers between bundles to provide air circulation. Protect materials with waterproof covering.

        - Do not store packaging materials in direct sunlight to prevent heat build up.

      - FIELD CONDITIONS

        - Weather Limitations: Proceed with installation only when existing and forecast weather conditions permit work to be performed.

        - Do not install PVC materials that are damaged.

      - WARRANTY

        - Manufacturer's Warranty for Cellular PVC Railings Systems: Manufacturer agrees to replace components that fails due to defects in manufacturing within specified warranty period.

          - Warranty Period: Limited Lifetime Warranty. Refer to manufacturer’s website for details.

- PRODUCTS

      - MANUFACTURER

        - INTEX Millwork Solutions, LLC; 20 Bogden Blvd., Millville, NJ 08332; Tel: (856) 293-4100, Fax: (856) 293-4102.

      - HAMPTON EXTRUDED RAIL SYSTEM

        - Cellular PVC: Extruded, expanded PVC with a small-cell microstructure, recommended by manufacturer for exterior use, made from UV- and heat-stabilized, rigid material.

          - Basis of Design Product: Subject to compliance with requirements, provide the following:

            - Intex Millwork Solutions; Hampton Extruded Rail System.

        - Top Rail Base: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Bottom Rail: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Aluminum Reinforcing Insert: 1 inch high by 1-3/4 inch wide 6063-T5 extruded aluminum “C” section (0.12 inch thick web; 0.06 in thick flanges) with four raceway channels running the entire length. Used in top and bottom rail of all systems.

        - Balusters: Model # RS40BAL-42. 1-1/4 in square extruded rigid cellular PVC pickets.

        - Newel Caps and Trim: Model # RS40PYCAP5 (Pyramidal cap), and #RS40BT5-WM75 (Base trim ring).

        - Decorative Panels: Manufacturer’s [standard] [custom] design.

        - Support Block: 1-1/4 in square extruded rigid cellular PVC picket cut to length and secured to the underside of the bottom rail.

        - Rail to Post Connection: Manufacturer’s standard.

        - Support Post: Preservative-treated wood (Southern Pine) 4 by 4 inches.

      - DARTMOUTH EXTRUDED RAIL SYSTEM

        - Cellular PVC: Extruded, expanded PVC with a small-cell microstructure, recommended by manufacturer for exterior use, made from UV- and heat-stabilized, rigid material.

          - Basis of Design Product: Subject to compliance with requirements, provide the following:

            - Intex Millwork Solutions; Dartmouth Extruded Rail System.

        - Top Rail Base: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Bottom Rail: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Aluminum Reinforcing Insert: 1 inch high by 1-3/4 inch wide 6063-T5 extruded aluminum “C” section (0.12 inch thick web; 0.06 in thick flanges) with four raceway channels running the entire length. Used in top and bottom rail of all systems.

        - Balusters: Model # RS40BAL-42. 1-1/4 in square extruded rigid cellular PVC pickets.

        - Newel Caps and Trim: Model # RS40PYCAP5 (Pyramidal cap), and #RS40BT5-WM75 (Base trim ring).

        - Decorative Panels: Manufacturer’s [standard] [custom] design.

        - Column Wraps: Manufacturer’s “Flat Panel” in height and width as indicated on Drawings.

        - Support Block: 1-1/4 in square extruded rigid cellular PVC picket cut to length and secured to the underside of the bottom rail.

        - Rail to Post Connection: Manufacturer’s standard.

        - Support Post: Preservative-treated wood (Southern Pine) 4 by 4 inches.

      - LIBERTY EXTRUDED RAIL SYSTEM

        - Cellular PVC: Extruded, expanded PVC with a small-cell microstructure, recommended by manufacturer for exterior use, made from UV- and heat-stabilized, rigid material.

          - Basis of Design Product: Subject to compliance with requirements, provide the following:

            - Intex Millwork Solutions; Dartmouth Extruded Rail System.

        - Top Rail Base: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Bottom Rail: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Aluminum Reinforcing Insert: 1 inch high by 1-3/4 inch wide 6063-T5 extruded aluminum “C” section (0.12 inch thick web; 0.06 in thick flanges) with four raceway channels running the entire length. Used in top and bottom rail of all systems.

        - Balusters: Model # RS40BAL-42. 1-1/4 in square extruded rigid cellular PVC pickets.

        - Newel Caps and Trim: Model # RS40PYCAP5 (Pyramidal cap), and #RS40BT5-WM75 (Base trim ring).

        - Decorative Panels: Manufacturer’s [standard] [custom] design.

        - Column Wraps: Manufacturer’s “Flat Panel” in height and width as indicated on Drawings.

        - Support Block: 1-1/4 in square extruded rigid cellular PVC picket cut to length and secured to the underside of the bottom rail.

        - Rail to Post Connection: Manufacturer’s standard.

        - Support Post: Preservative-treated wood (Southern Pine) 4 by 4 inches.

      - NAUTILUS MILLED RAIL SYSTEM

        - Cellular PVC: Extruded, expanded PVC with a small-cell microstructure, recommended by manufacturer for exterior use, made from UV- and heat-stabilized, rigid material.

          - Basis of Design Product: Subject to compliance with requirements, provide the following:

            - Intex Millwork Solutions; Nautilus Milled Rail System.

        - Top Rail Base: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Bottom Rail: 1-1/2 inch high by 2-15/16 inch wide extruded rigid cellular PVC contoured rail profile.

        - Aluminum Reinforcing Insert: 1 inch high by 1-3/4 inch wide 6063-T5 extruded aluminum “C” section (0.12 inch thick web; 0.06 in thick flanges) with four raceway channels running the entire length. Used in top and bottom rail of all systems.

        - Balusters: Model # RS40BAL-42. 1-1/4 in square extruded rigid cellular PVC pickets.

        - Newel Caps and Trim: Model # RS40PYCAP5 (Pyramidal cap), and #RS40BT5-WM75 (Base trim ring).

        - Decorative Panels: Manufacturer’s [standard] [custom] design.

        - Column Wraps: Manufacturer’s “Flat Panel” in height and width as indicated on Drawings.

        - Support Block: 1-1/4 in square extruded rigid cellular PVC picket cut to length and secured to the underside of the bottom rail.

        - Rail to Post Connection: Manufacturer’s standard.

        - Support Post: Preservative-treated wood (Southern Pine) 4 by 4 inches.

      - MISCELLANEOUS MATERIALS

        - Fasteners for PVC Railing Components: Provide manufacturer’s recommended fasteners.

          - (A) Rail Bracket Screws: #8 x 1-1/4 inches Flat Head Square Drive.

          - (B) Rail Attachment Screws: #10 x 3 inches Slot Hex Washer Head.

          - Top Rail Cap Attachment Screws:

            - (C) #8 x 1-3/4 inches Flat Head Square Drive (RS40 Flat Cap ONLY).

            - (D) #8 x 2-1/4 inches Flat Head Square Drive (RS40 Peaked Cap ONLY).

          - (E) Baluster Screws: #8 x 2-1/2 inches Flat Head Square Drive.

          - (F) Baluster Lock Screws: #8 x 1-1/2 inches Flat Head Square Drive.

          - (G) Rail Attachment Screws: #12 x 4 inches Slot Hex Washer Head. For level sections greater than 8 ft. long and all stair rails.

        - Adhesive for Cellular PVC: Product recommended by manufacturer.

        - Sealants: Type as recommended by manufacturer and complying with ASTM C 834 and with applicable requirements in Division 07 Section "Joint Sealants."

- EXECUTION

      - EXAMINATION

        - Examine substrates, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance.

        - Examine PVC materials before installation. Reject materials that are damaged.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Clean substrates of projections and substances detrimental to application.

      - INSTALLATION, GENERAL

        - Do not use materials that are unsound or warped.

          - Do not use manufactured units with defective surfaces, sizes, or patterns.

        - Install exterior finish carpentry level, plumb, true, and aligned with adjacent materials.

          - Scribe and cut PVC components to fit adjoining work.

          - Coordinate PVC components with materials and systems in or adjacent to it. Provide cutouts for mechanical and electrical items that penetrate exterior finish carpentry.

      - INSTALLATION - GENERAL

        - Install PVC railing systems in strict accordance with manufacturer’s written installation instructions, and detailed shop drawings.

        - Refer to manufacturer’s website for latest information and installation videos.

      - ADJUSTING

        - Replace PVC components that is damaged or does not comply with requirements. Adjust joinery for uniform appearance.

      - CLEANING

        - Clean exposed and semiexposed PVC surfaces.

      - PROTECTION

        - Protect installed products from damage from weather and other causes during construction.

        - Remove and replace PVC materials that are damaged.

END OF SECTION